Sika Sarnafil®

Green Roof –

Extensive

Adhered System

Waterproofing

Specification

Conventional Assembly

DISCLAIMER

All information provided by Sika Corporation (“Sika”) concerning Sika products, including but not limited to, any recommendations and advice relating to the application and use of Sika products, is given in good faith based on Sika’s current experience and knowledge of its products when properly stored, handled and applied under normal conditions in accordance with Sika’s instructions. In practice, the differences in materials, substrates, storage and handling conditions, actual site conditions and other factors outside of Sika’s control are such that Sika assumes no liability for the provision of such information, advice, recommendations or instructions related to its products, nor shall any legal relationship be created by or arise from the provision of such information, advice, recommendations or instructions related to its products. The user of the Sika product(s) must test the product(s) for suitability for the intended application and purpose before proceeding with the full application of the product(s). Sika reserves the right to change the properties of its products without notice. All sales of Sika product(s) are subject to its current terms and conditions of sale which are available at usa.sarnafil.sika.com or by calling 800-451-2504.

Prior to each use of any Sika product, the user must always read and follow the warnings and instructions on the product’s most current Product Data Sheet, product label and Safety Data Sheet which are available online at usa.sarnafil.sika.com or by calling Sika's Technical Service Department at 800-451-2504. Nothing contained in any Sika materials relieves the user of the obligation to read and follow the warnings and instruction for each Sika product as set forth in the current Product Data Sheet, product label and Safety Data Sheet prior to product use.

Sika warrants this product for one year from date of installation to be free from manufacturing defects and to meet the technical properties on the current Product Data Sheet if used as directed within shelf life. User determines suitability of product for intended use and assumes all risks. Buyer’s sole remedy shall be limited to the purchase price or replacement of product exclusive of labor or cost of labor.

NO OTHER WARRANTIES EXPRESS OR IMPLIED SHALL APPLY INCLUDING ANY WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE. SIKA SHALL NOT BE LIABLE UNDER ANY LEGAL THEORY FOR SPECIAL OR CONSEQUENTIAL DAMAGES. SIKA SHALL NOT BE RESPONSIBLE FOR THE USE OF THIS PRODUCT IN A MANNER TO INFRINGE ON ANY PATENT OR ANY OTHER INTELLECTUAL PROPERTY RIGHTS HELD BY OTHERS.

Introduction

Self-Adhered Sheet Waterproofing System For Extensive Green Roofs	2

System Description	2

System Components	2

Regional Offices	4

Part 1 – General Conditions

1.01 Description	1

1.02 Quality Assurance	1

1.03 Submittals	2

1.04 Product Delivery, Storage and Handling	2

1.05 Job Conditions	2

1.06 Sequencing of the Work	4

1.07 Bidding Requirements	4

1.08 Warranties	4

Part 2 - Products

2.01 Waterproofing Membrane	5

2.02 System Flashing Products and Accessories	5

2.03 Vapor Barrier	5

2.04 Insulation/Thermal Barrier/Cover Board	6

2.05 Vector Mapping Grid and Connections	6

2.06 Attachment Components	6

2.07 Separation Layer	8

2.08 Drainage Composite	9

2.09 Precast Concrete Pavers and Pedestals	9

2.01 Related Materials	9

Part 3 – Execution

3.01 Examination	10

3.02 Substrate Preparation	10

3.01 Wood Nailer Installation	11

3.02 Thermal Barrier Installation	12

3.03 Vapor Barrier/Air Barrier Installation	12

3.04 Insulation Installation	12

3.01 Vector Mapping Grid and Connections/Cover Board Installation	13

3.02 Surface Conditioner/Primer Application	13

3.03 Self-Adhered Thermoplastic Waterproofing Membrane Installation	13

3.04 Hot-Air Welding of Lap Areas	14

3.05 Membrane Flashings	14

3.06 Sarnaclad Metal Flashings	15

3.07 Temporary Cut-Off	15

3.08 Water Testing of Drains and Electronic Leak Detection Testing	15

3.09 Separation Layer Installation	16

3.10 Drainage Composite Installation	16

3.11 Precast Concrete Paver System	16

3.12 Monitoring the Vegetative Cover Installation	16

3.13 Details	17

INTRODUCTION TO

SIKA SARNAFIL SELF-ADHERED SHEET WATERPROOFING SYSTEM

FOR EXTENSIVE GREEN ROOFS

System Description

The Sika Sarnafil Self-Adhered Waterproofing System (Conventional Assembly) is designed to protect structures from the effects of water infiltration leading to structural deterioration and interior water damage.  The system configuration includes insulation and cover board below the waterproofing membrane making it ideal for extensive green roofs (6 inches (152 mm) or less of growth media) installed over steel deck, but can be used over almost any structural deck.

The Self-Adhered System design is intended to provide increased security by having the waterproofing membrane bonded directly to the substrate.  The waterproofing membrane is a composite sheet comprised of a waterproofing membrane and a closed cell foam backing layer coated with a pressure-sensitive adhesive.  The backing layer allows the membrane to better conform to minor surface irregularities, and acts as a separation between the substrate and waterproofing membrane.  The pressure-sensitive adhesive is designed to adhere to properly prepared substrates.

There are two design options depending on the type of insulation selected.  Assembly 1 utilizes polyisocyanurate insulation, and Assembly 2 utilizes extruded polystyrene insulation.

System Components

ASSEMBLY 1

The Self-Adhered System Assembly 1 design includes the following:

#### Vapor Barrier (if specified).

#### Sarnatherm Polyisocyanurate Insulation.

#### Vector Mapping Grid for Electronic Leak Detection Testing.

#### DensDeck® Prime Cover Board.

#### Sarnafil G476 Self-Adhered Waterproofing Membrane.

#### Drainage Layer.

#### Filter Fabric (optional).

ASSEMBLY 2

The Self-Adhered System Assembly 2 design includes the following:

#### Thermal Barrier.

#### Vapor Barrier (if specified).

#### Sarnatherm Extruded Polysytrene Insulation.

#### Vector Mapping Grid for Electronic Leak Detection Testing.

#### DensDeck® Prime Cover Board.

#### Sarnafil G476 Self-Adhered Waterproofing Membrane.

#### Drainage Layer.

#### Filter Fabric (optional).

REGIONAL OFFICES

| NEW ENGLAND REGION 225 Dan Road Canton, MA 02021 Phone:(781) 821-0865 Fax:(781) 821-9205 EASTERN REGION One Park Way 3rd Floor Upper Saddle River, NJ 07458 Phone:(201) 327-0479 Fax:(201) 327-4069 SOUTHERN REGION 3483 Satellite Boulevard Duluth, GA 30096 Phone:(770) 495-0025 Fax:(770) 495-0027 MIDWEST REGION 200 W. 22nd St., Suite 216 Lombard, IL 60148 Phone:(800) 532-5123 Fax:(630) 620-9646 | SOUTHWEST REGION 2517 Fairway Park, Suite 200 Houston, TX 77092 Phone: (713) 812-0102 Fax: (713) 812-0107 MOUNTAIN REGION 2881 South 900 West Salt Lake City, UT 84119 Phone:(801) 575-8648 Fax:(801) 355-4407 WESTERN REGION NORTH 2375 Rodolfo Court Sparks, NV 89436 Phone:(775) 626-7701 Fax:( 775) 626-7703 WESTERN REGION SOUTH 6590 Darin Way Cypress, CA 90630 Phone:(714) 898-9355 Fax:(714) 898-9357 |
| --- | --- |
| WEB ADDRESS: www.sikacorp.com | EMAIL ADDRESS: webmaster.sarnafil@us.sika.com |
| SIKA CANADA - ROOFING 6915 Davand Drive Mississauga, ON L5T 1L5 Phone:(905) 795-3177 Fax:(905) 795-3192 www.sika.ca |  |

SECTION 07 54 19

THERMOPLASTIC MEMBRANE WATERPROOFING

SELF-ADHERED SYSTEM

[NOTE TO SPECIFIER:  NOTES TO SPECIFIER ARE DESIGNATED BY [ ].  SPECIFIER IS TO SELECT ONE OF THE OPTIONS PROVIDED FOR PROJECT SPECIFIC SPECIFICATIONS.]

TO DISPLAY OR HIDE SPECIFIER NOTES

MS Word (2007 and later):	MS Word (prior to 2007):

1. Select the OFFICE logo or FILE in the upper left corner.	1. Select TOOLS.

2. Select OPTIONS.	2. Select OPTIONS.

3. Select DISPLAY on the left menu.	3. Select VIEW.

4. Select HIDDEN TEXT under “Always Show These”.	4. Select HIDDEN TEXT.

˙

This guide specification is intended for extensive green roof applications.

# GENERAL CONDITIONS

## DESCRIPTION

- Scope

To install an adhered thermoplastic membrane waterproofing system with integral flashings and other components.

The work includes, but is not limited to, the following:

#### Substrate Preparation

#### Thermal Barrier (If Specified)

#### Vapor Barrier (If Specified)

#### Insulation

#### Vector Mapping Grid and Connections

#### Cover Board

#### Waterproofing Membrane

#### Membrane Flashings

#### Metal Flashings

#### Drainage Composite (If Specified)

#### Sealants and Adhesives

#### Monitoring the Installation of Vegetative Cover

#### Pavers (If Specified)

- Related Work Under Other Sections

[NOTE TO SPECIFIERS:  SPECIFY OTHER RELATED SECTIONS SUCH AS VEGETATIVE COVER, CONTRACT DOCUMENTS, SITE WORK, PLUMBING, ETC.]

## QUALITY ASSURANCE

- This waterproofing system shall be applied only by a waterproofing Applicator authorized by Sika Sarnafil prior to bid. All work shall be completed by Sika Sarnafil trained personnel.

- Installation of all waterproofing system components up to and including the waterproofing membrane shall be the responsibility of the waterproofing applicator to ensure undivided responsibility.

- Obtain primary waterproofing, membrane and flashings, from a single manufacturer with not less than 20 years of successful experience in waterproofing applications.  Provide other system components only as approved by manufacturer of primary materials.

- Pre-construction conference to be held with the owner, architect, Applicator’s field superintendent, waterproofing foreman, Sika Sarnafil representative, landscaper, and other involved trades to discuss waterproofing practices applicable to the project.

- There shall be no deviation made from the contract specification or the approved shop drawings without prior written approval by the owner or the owner’s representative, and/or design professional, and Sika Sarnafil.

- The Applicator shall follow Sika Sarnafil’s most current quality assurance procedures.

- Cross-section seam samples shall be taken by the applicator a minimum of two times a day (AM/PM) through completed seams and evaluated immediately.  The samples must be dated and saved for evaluation by a Sika Sarnafil Technical Representative.  Each test cut shall be patched by the Applicator.

- Membrane Testing for Water Tightness

#### Water Testing of Drains

The Applicator shall water test drains according to Sika Sarnafil’s most current Quality Assurance Procedures. See Section 3.14.

### [NOTE TO SPECIFIERS:  ELECTRONIC LEAK DETECTION MUST BE CONDUCTED ON THE COMPLETED WATERPROOFING MEMBRANE FOR QUALITY ASSURANCE PURPOSES WHEN A STANDARD, SYSTEM, OR SINGLE-SOURCE WARRANTY IS SPECIFIED. (THE TEST IS RECOMMENDED, BUT NOT REQUIRED, FOR MEMBRANE WARRANTIES.)  SELECT ONE OF THE FOLLOWING DEPENDING ON THE WARRANTY. SEE TECHNICAL BULLETIN #15-7 FOR IMPORTANT INFORMATION CONCERNING THE USE OF ELECTRONIC LEAK DETECTION FOR LOCATING LEAKS THROUGH OVERBURDEN.]

˙

#### Electric Field Vector Mapping® (EFVM®)

#### Electronic Leak Detection Testing (System, or Single-Source Warranty) – The waterproofing applicator shall arrange for testing through Sika Sarnafil.  See Section 3.084.

#### Electronic Leak Detection Testing (Membrane Warranty) – The waterproofing applicator shall arrange for testing by a qualified testing agency.  See Section 3.084.

## SUBMITTALS

### The Applicator shall submit to the owner’s representative and/or design professional the following:

#### A letter from Sika Sarnafil certifying that the Applicator is an approved waterproofing applicator in good standing.

#### Specimen copy of Sika Sarnafil’s warranty.

#### Specimen copy of Applicator’s warranty.

#### Sample of Sarnafil G476 Self-Adhered Waterproofing Membrane.

## PRODUCT DELIVERY, STORAGE AND HANDLING

- All products delivered to the job site shall be in the original unopened containers or wrappings.

- Handle all materials to prevent damage.  All materials shall be placed on pallets and fully protected from the elements with canvas tarpaulins.

- Membrane rolls shall be stored lying down on pallets and fully protected from moisture with canvas tarpaulins.

- Membrane, adhesives, and surface conditioner shall be stored at temperatures above 40 degree F (5 degree C).

- All flammable materials shall be stored in a cool dry area away from sparks and open flames.  Follow precautions outlined on container or supplied by the material manufacturer/supplier.

- Any materials which the owner’s representative and/or Sika Sarnafil determine to be damaged are to be removed from the job site and replaced at no cost to the owner.

## JOB CONDITIONS

- Proceed with waterproofing membrane installation only after substrate preparation is complete.  Owner’s representative and/or design professional and waterproofing Applicator must accept substrate before proceeding with membrane installation.

- Substrate must be clean, smooth and dry.  Do not work in rain or snow or adverse weather conditions.  Severe temperatures, moisture and humidity may affect the installation of products during construction.  Comply with applicable installation requirements for all components. Ambient and substrate temperature must meet the minimum requirements as outlined in the “Sarnafil G476 Self-Adhered Waterproofing System Installation Instructions.”

- All work shall be scheduled and executed without exposing the completed waterproofing system and the building interior to the affects of inclement weather.  The building and its contents shall be protected against all risks.

- The structure must be designed to support the system, including the vegetative cover.  The adequacy of the structural support must be verified in writing by the owner, the owner’s design professional, architect, or engineer. The Applicator shall take precautions that storage and/or application of materials and/or equipment does not overload the deck or building structure.

- All new and temporary construction, including equipment and accessories, shall be secured in such a manner, at all times, as to preclude wind blow-off or damage.

- Liquid materials such as solvents and adhesives shall be stored and used away from open flames, sparks and excessive heat.

- The Applicator should take necessary precautions when using adhesives or surface conditioner around air in-takes.  The smell of the adhesive, primer, or surface conditioner could be a disturbance to the building owner and occupants.  It is the Applicator’s responsibility to notify the owner and take the proper precautions.

- The Applicator shall verify that all drain lines are connected and un-blocked before starting work.  Report any blockages or non-connected drains to the owner’s representative and/or design professional in writing.

### The Applicator is cautioned that Sarnafil membranes are incompatible with asphalt, coal-tar, polystyrene, oil-based and plastic-based cements, creosote, penta-based materials, grease, fats, oils, and solvents. Such materials shall not come in contact with the waterproofing membrane at any time.  If such contact occurs, the material shall be cut-out, discarded and patched.

- Arrange work sequence to avoid use of newly installed waterproofing for storage, walking surface, and equipment movement.  Where such access is absolutely required, the Applicator shall provide all necessary temporary protection and barriers to segregate the work area and to prevent damage to adjacent areas.  Adequate protection of the membrane shall be provided for all waterproofing areas which receive traffic during construction.  Any damage which occurs to the waterproofing membrane and/or system is to be brought to the attention of the owner’s representative and/or design professional and Sika Sarnafil.  All damage is to be repaired according to Sika Sarnafil’s recommendations.  The party responsible for damage shall bear the cost of repairs.

- All waterproofing materials, insulation, flashings and metal work removed for construction shall be immediately taken off the site to a legal dumping area authorized to receive such materials.

- If any unusual or concealed condition is discovered, stop work and notify the owner’s representative and/or design professional and Sika Sarnafil immediately, in writing.

- Site cleanup, including both interior and exterior building areas in any way affected by the construction, shall be complete and to the owner’s satisfaction. All landscaped areas affected by waterproofing activities shall be raked clean and seeded, if required. All paved areas shall be swept clean. All areas stained, dirtied, and discolored or otherwise damaged due to waterproofing activities shall be cleaned, restored, and replaced as required.

## SEQUENCING OF THE WORK

#### Do not proceed with installation of vegetative cover over the completed sections of the waterproofing without the acceptance of the owner’s representative, design professional, general contractor, and Sika Sarnafil (when a Standard, System, or Single-Source warranty is specified).  A copy of the Final Inspection for Warranty is considered acceptance from Sika Sarnafil.

#### Protect the membrane and coordinate with other trades to avoid traffic over completed membrane surfaces.

## BIDDING REQUIREMENTS

[NOTE TO SPECIFIERS:   DELETE THIS SECTION FOR NEW CONSTRUCTION PROJECTS.]

### Pre-Bid Meeting:

A pre-bid meeting shall be held with the Owner's Representative and involved trades to discuss all aspects of the project.  The Applicator's field representative or roofing foreman for the work shall be in attendance.  Procedures to avoid rooftop damage by other trades shall be determined.

### Site Visit:

Bidders shall visit the site and carefully examine the areas in question as to conditions that may affect proper execution of the work.  All dimensions and quantities shall be determined or verified by the contractor.  No claims for extra costs will be allowed because of lack of full knowledge of the existing conditions unless agreed to in advance with the Owner or Owner's Representative.

## WARRANTIES

- General

#### A Sika Sarnafil Representative’s presence on a project regardless of reason, length, or frequency, does not imply that any additional coverage beyond that stated in the warranty is in effect.

- Applicator’s Warranty

The Applicator shall supply the owner with a minimum two-year workmanship warranty. The warranty obligation shall run directly to the owner with a copy to Sika Sarnafil.

- Sika Sarnafil Warranty

[NOTE TO SPECIFIERS:  SELECT THE WARRANTY TYPE AND LENGTH BELOW.]

#### Membrane Warranty (5, 10, 15, or 20 Years)

#### The Applicator shall provide a Sika Sarnafil membrane warranty to the building owner at the successful completion of the project.  The warranty shall cover defective waterproofing membrane.

#### System Warranty (5, 10, 15, or 20 Years)

The Applicator shall provide a Sika Sarnafil System warranty to the building owner. Upon successful completion of the work to Sika Sarnafil's satisfaction and receipt of final payment, the Sika Sarnafil System Warranty shall be issued.

#### Single-Source Warranty – Extensive Green Roofs (5, 10, 15, or 20 Years)

[NOTE TO SPECIFIERS:  THIS WARRANTY APPLIES ONLY TO GREEN ROOFS WITH A GROWTH MEDIA DEPTH OF 6 INCHES OR LESS. THIS WARRANTY REQUIRES THE OWNER TO ARRANGE FOR A GREEN ROOF MAINTENANCE PROGRAM FOR THE LIFE OF THE WARRANTY.]

The Applicator shall provide a Sika Sarnafil Single-Source warranty to the building owner at the successful completion of the project.  The waterproofing membrane manufacturer shall warrant to the building owner the vegetative cover, and the repair of leaks in the waterproofing membrane resulting from defects in the membrane or workmanship.

# PRODUCTS

General

Consult Product Data Sheet for additional information on Sika Sarnafil products.

##### WATERPROOFING MEMBRANE

### The manufacturer of the thermoplastic waterproofing membrane system shall have a track record of producing and marketing a reinforced PVC sheet system for waterproofing applications for at least 20 years and guarantee that the membrane thickness meets or exceeds [the specified thickness] when tested according to ASTM D751.  Subject to this and other technical requirements, provide products of the following:

  - Sika Sarnafil, A Division of Sika Corporation, Canton, Massachusetts

Self-adhered thermoplastic membrane: polyvinyl chloride (PVC) flexible sheets with non-woven fiberglass reinforcing and closed-cell foam backing layer.

#### [NOTE TO SPECIFIERS:  DO NOT USE WATERPROOFING MEMBRANE IN SITUATIONS WHERE THE MEMBRANE WILL BE PERMANENTLY EXPOSED TO SUNLIGHT.]

Sarnafil G476-15 SA Waterproofing Membrane – a self-adhered, cold-applied composite sheet consisting of a 60 mil (1.5 mm), thermoplastic waterproofing membrane with fiberglass reinforcement and 60 mil (1.5 mm) closed-cell foam backing layer having an overall thickness of 120 mils (3 mm).  The backing layer has a factory-applied pressure sensitive adhesive that is protected by a release liner which is removed during application.  Membrane has a selvedge to allow a heat-welded overlap of adjoining sheets.

Parameters	ASTM Test Method	Typical Physical Properties

Membrane thickness, inches (mm)	D 638	0.060 (1.5)

Tensile Strength, psi.	D 751	1631

Elongation at Break, %	D 751	279 MD, 267 CMD

Seam Strength, % of Tensile Strength	D 751	86%

Retention of Properties After Heat Aging	D 3045

Tensile Strength, % of original	D 751	90%

Elongation at Break, % of original	D 751	90%

Tear Resistance, lbf	D 1004	21.0

Linear Dimensional Change, %	D 1204	0.0

Weight Change after immersion in water, %	D570	1.5

Static Puncture Resistance, 33 lbs. 	D5602	Pass

Dynamic Puncture Resistance, 10 J	D5635	Pass

##### SYSTEM FLASHING PRODUCTS AND ACCESSORIES

- Flashing

#### Non-exposed flashings

                - Sarnafil G476-15 SA Waterproofing Membrane.

                - Sarnafil G476-15, 60 mil (1.5 mm), thermoplastic flashing membrane with fiberglass reinforcement.

#### Exposed flashings

  - Sarnafil G410-15, 60 mil (1.5 mm), thermoplastic flashing membrane with fiberglass reinforcement.

  - G459-15, 60 mil (1.5 mm), thermoplastic, asphalt resistant, grid strip and flashing membrane with fiberglass reinforcement.

##### VAPOR BARRIER

### Sarnavap-10

A 10 mil (0.25 mm) thick polyethylene vapor retarder/air retarder.

### Sarnavap Self-Adhered

A 32 mil (0.8 mm) self-adhesive vapor barrier that can also serve as temporary roof protection.

##### INSULATION/THERMAL BARRIER/COVER BOARD

[NOTE TO SPECIFIERS:  SELECT ONLY ONE TYPE OF INSULATION TO SPECIFY.]

˙

### Sarnatherm Polyisocyanurate

#### A rigid polyisocyanurate foam insulation with black mat facers.  Thicknesses and compressive strength as noted on Project Drawings.

### Sarnatherm XPS

Extruded polystyrene closed-cell foam insulation.  Thicknesses and compressive strength as noted on Project Drawings.

[NOTE TO SPECIFIERS:  A THERMAL BARRIER MAY BE REQUIRED BY CODE UNDER SARNATHERM XPS IN CERTAIN ROOF CONSTRUCTIONS.]

˙

### DensDeck® Thermal Barrier

A siliconized gypsum, fire-tested roof board with glass-mat facer used as a thermal barrier underlayment in certain roof constructions.  Thicknesses as noted on Project Drawings.

[NOTE TO SPECIFIERS:  FOR THERMAL BARRIER USE ONLY. DO NOT USE AS A COVER BOARD FOR ADHERED MEMBRANES.]

˙

### DensDeck® Prime Cover Board

A 1/4 inch (6 mm) thick fire-tested, gypsum cover board with glass-mat facers and a pre-primed surface on one side as a substrate for adhered membrane.

##### Vector Mapping Grid and Connections

### Vector Mapping Grid (VMG)

### A specially selected 2” x 2” (50 mm x 50 mm) 304 stainless steel grid that serves as a conductive medium below the membrane used in conjunction with EFVM.

### EFVM Connection Kit

### Connects the Vector Mapping Grid to the leak detection equipment. It consists of two (2) 0.45 mm, 6” x 6” SS 304 stainless steel EFVM connection plates, insulated, exterior graded 16 gauge low voltage wire, 5’-6” in length.  The EFVM Connection Kit is utilized to electrically energize the Vector Mapping Grid during the EFVM test.

##### ATTACHMENT COMPONENTS

### Insulation Board Adhesive

#### Sarnacol 2163 Adhesive

A low odor, VOC compliant, two-component, one step, low-rise urethane foam used to attach insulation to approved compatible substrates.  Adhesive is applied in ribbons with a gravity fed applicator or by hand with a dual component caulk gun.

#### Olympic Olybond 500 Adhesive

A two-component (Part A and B) low-rise polyurethane foam used to attach insulation to approved compatible substrates.  Adhesive is applied with a specially designed dispensing cart in bands 12 inches (300 mm) on center.

### Fastening Components

#### Sarnaplate-Low Profile

A 2-3/4 inch (70 mm) square steel low profile plate used with various Sarnafasteners to attach insulation boards to roof deck.

#### Sarnaplate

A 3 inch (75 mm) square or round steel plate used with various Sarnafasteners to attach insulation boards to roof deck.

#### Sarnafastener #12

A #12 fastener used with Sarnaplate to attach insulation boards to steel or wood roof decks.

#### Sarnafastener #14

A #14 fastener used with Sarnaplate to attach insulation boards to structural concrete or wood roof decks.

#### Sarnafastener-XP

A #15 fastener used with Sarnaplate to attach insulation to steel roof decks.

#### Sarnafastener-CD10

A nail-in fastener used with Sarnaplate to attach insulation to concrete roof decks.

#### Sarnafastener #12 Preassembled

#### A preassembled 3 inch round plate and #12 fastener used to attach insulation boards to steel or wood roof decks.

#### Masonry Anchor

#### A drive-pin expansion type fastener with zinc sheaths and stainless steel pins for attachment of Sarnastop to concrete, masonry, and brick.

#### Sarnastop

#### An extruded aluminum, flat low profile bar used to terminate flashing membrane.

### Adhesives

#### Sarnacol 2170

Solvent-based adhesive for membrane flashings in areas where self-adhered membrane is not used.

#### Sarnacol 2121

Water-based adhesive for membrane flashings in areas where self-adhered membrane is not used.

#### Sikaflex-11FC

#### Membrane adhesive for certain flashing details.

### Accessories

#### Surface Conditioner 150

For concrete and masonry substrates only.  A water-based surface treatment specifically designed to bind dust and concrete efflorescence thereby providing a suitable surface to install self-adhered membrane. For use when substrate and ambient temperature is 40 degree F (5 degree C) or higher.

#### Sarnavap Self-Adhered Primer

For use on all substrates.  A solvent-based surface treatment specifically designed to bind dust and concrete efflorescence thereby providing a suitable surface to install self-adhered membrane. For use when substrate and ambient temperature is 25 degree F (-4 degree C) or higher.

#### Sikaflex Primer 449

A solvent-based primer used to prime the back of G459 grid strip membrane to improve adhesion to Sikaflex-11FC.

#### Sarnaclad-SS

A PVC-coated, heat weldable sheet metal capable of being formed into a variety of shapes and profiles.  Sarnaclad-SS is a stainless steel sheet metal with an unsupported Sarnafil membrane laminated on one side for buried or concealed flashing conditions.

#### Sarnaclad

#### A PVC-coated, heat-weldable sheet metal capable of being formed into a variety of shapes and profiles.  Sarnaclad is a 25 gauge, G90 galvanized metal sheet with a 20 mil (0.5 mm) unsupported Sarnafil membrane laminated on one side for above grade flashing locations.

#### Aluminum Tape

A pressure-sensitive aluminum tape used as a separation layer between small areas of asphalt contamination and as a bond-breaker under the cover strip at Sarnaclad joints.

#### Sarnamatic

220 volt, self-propelled, hot-air welding machine used to seal long lengths of Sarnafil membrane seams.

#### Sarnacorner-Inside/Outside

Prefabricated inside and outside flashing corners made of PVC membrane that are heat-welded to membrane or Sarnaclad base flashings.

#### Sikaflex®-1a

Sealant used at flashing terminations and pitch pocket filler.

#### Multi-Purpose Tape

#### A high performance sealant tape used with metal flashings as a preventive measure against air and wind blown moisture entry.

#### Sarnacircle

Circular 48 mil (1.2 mm) G410 membrane patch welded over T-joints formed by overlapping membranes.

##### SEPARATION LAYER (If Specified)

### Sarnafelt NWP

A non-asphaltic, non-woven polypropylene felt separation layer used between Vector Mapping Grid (VMG) and waterproofing membrane, or used above the membrane as an additional protective layer as required.

##### DRAINAGE COMPOSITE

[NOTE TO SPECIFIERS:  DELETE THIS SECTION IF DRAINAGE IS INCLUDED IN THE VEGETATIVE COVER SECTION.]

[NOTE TO SPECIFIERS:  SELECT A DRAINAGE COMPOSITE FROM THE CHOICES BELOW. ALWAYS SELECT A. IF SPECIFYING PRE-VEGETATED MODULES (I.E.TRAYS).]

˙

### Drainage Composite 3811R

A 100% recycled polypropylene drainage core of fused, entangled filaments with a geotextile fabric bonded to each side.

### Geonet B

A polymeric drainage net with polypropylene geotextile laminated to both sides. Recommended for heavy traffic areas requiring higher compressive strength but where lower flow rates are acceptable.

##### PRECAST CONCRETE PAVERS AND PEDESTALS

[NOTE TO SPECIFIERS:  SELECT ONE OF THE FOLLOWING.]

### Pavers shall be hydraulically pressed concrete units manufactured by Hanover Architectural Products or Oldcastle/Westile Inc., and supplied by Sika Sarnafil.

#### Precast Concrete Pavers and Pedestal System by Hanover Architectural Products and supplied by Sika Sarnafil.

#### Precast Concrete Pavers and Pedestal System by Oldcastle/Westile Inc. and supplied by Sika Sarnafil.

#### Concrete pavers shall comply with the following minimum requirements:

| Physical Property | ASTM Test Method | Requirements |
| --- | --- | --- |
| Dimensional Tolerances | C140 | As specified ±1/8 inch maximum |
| Compressive Strength | C140 | 8000 psi minimum |
| Flexural Strength | C293 | 725 psi minimum |
| Freeze Thaw Resistance | C67 | 25 cycles, max. 1% loss |
| Water Absorption | C140 | 5% or less |

[NOTE TO SPECIFIERS:  CONSULT HANOVER OR OLDCASTLE/WESTILE PRODUCT DATA SHEETS AND LITERATURE FOR AVAILABLE DIMENSIONS, COLORS, FINISHES, AND PEDESTAL SYSTEMS.]

˙

#### Precast Concrete Paver thickness shall be: ______

#### Precast Concrete Paver finish shall be: ______

#### Precast Concrete Paver color shall be: ______

#### Precast Concrete Paver size shall be: ______

#### Pedestal System shall be: ______

##### RELATED MATERIALS

### Wood Nailer

Wood nailers shall be treated for fire and rot resistance (ACQ treated) and be #2 quality or better lumber.  Creosote or asphalt-treated wood is not acceptable.  Wood nailers shall conform to Factory Mutual Loss Prevention Data Sheet 1-49.  All wood shall have a maximum moisture content of 19% by weight on a dry-weight basis.

[NOTE TO SPECIFIERS:  WOOD NAILERS OR WOOD BLOCKING FOR SOIL RETENTION SHALL BE INSTALLED PRIOR TO THE INSTALLATION OF THE ROOF MEMBRANE WHENEVER POSSIBLE.]

˙

### Plywood

When adhering the flashing membrane directly to plywood, a minimum 1/2 inch (13 mm) CDX (C side out), smooth-surfaced exterior grade plywood with exterior grade glue shall be used.  Rough-surfaced plywood or high fastener heads will require the use of Sarnafelt behind the flashing membrane.  Plywood shall have a maximum moisture content of 19% by weight on a dry weight basis.

### Miscellaneous Fasteners and Anchors

Fasteners are to be compatible with materials in contact with fasteners.  All fasteners and anchors shall have a minimum embedment of 1-1/4 inches (31.7 mm) and shall be approved for such use by the fastener manufacturer.  Fasteners for attachment of metal to wood blocking shall be annular ring nails.  Fasteners for attachment of metal to masonry shall be all-metal expansion type fasteners.  All fasteners shall meet Factory Mutual Standard 4470 for corrosion resistance.

# EXECUTION

###### EXAMINATION

### Examine all surfaces scheduled to receive waterproofing membrane and flashing for roughness, contaminants, unsound structural substrates or other conditions that may impair the waterproofing application.  Notify the owner and copy Sika Sarnafil in writing of any such conditions. Do not commence work until all defects are remedied.

### Applicator shall be responsible for acceptance or provision of proper substrate to receive new waterproofing materials.

### Applicator shall verify that the work done under related sections meets the following conditions:

#### Roof drains and/or scuppers have been reconditioned and/or replaced and installed properly.

#### Roof curbs, nailers, equipment supports, vents and other roof penetrations are properly secured

#### and prepared to receive new waterproofing materials.

#### All surfaces are smooth and free of dirt, debris and incompatible materials.

###### SUBSTRATE PREPARATION

[NOTE TO SPECIFIERS:  FROM THE CHOICES BELOW, SELECT THE APPROPRIATE SUBSTRATE PREPARATION.]

˙

### New Construction

#### Steel Deck:

The structural deck shall be minimum 24 gauge, grade D and shall conform and be installed according to the current local code requirements.

#### Wood Deck:

The structural deck shall be minimum 1-1/2 inch (38 mm) thick lumber or 15/32 inch (12 mm) thick plywood.  Deck shall be installed according to the current local code requirements.

#### Poured Structural Concrete Deck:

The structural deck shall be installed and cured in accordance with industry standards.  The surface shall have a smooth and level finish and shall be free of excess moisture and loose debris.  Sharp ridges or other projections above the surface shall be removed before installation of roofing assembly.

#### Precast/Prestressed Concrete Panel Deck:

The structural deck shall be installed in accordance with the concrete panel manufacturer's requirements and industry practice.  The surface shall have a smooth and level finish and shall be free of excess moisture and loose debris.  All joints between precast units shall be grouted with SikaGrout® 212 non-shrink, cementitious grout.  Any differentials in height between precast units shall be feathered for a smooth transition.  Sharp ridges or other projections above the surface shall be removed before installation of roofing assembly.

### Removal of Existing Waterproofing

General Criteria

All existing overburden, waterproofing, base flashing, deteriorated wood blocking or deteriorated metal flashings shall be removed.  Remove only that amount of waterproofing and flashing that can be made weathertight with new materials during a one-day period or before the onset of inclement weather.

Residual material of most products, including those that are asphalt based, may remain with the exception of coal tar.  All coal tar pitch residue must be completely removed.

#### Steel Deck:

- All rusted or deteriorated decking shall be brought to the attention of the Owner's Representative to determine method of treatment or replacement.  Surface-only rusted metal shall be sanded and treated with rust-inhibiting paint.  Sections that have rusted deeper than the surface or are not structurally sound shall be removed and replaced with matching deck type.

#### Wood Deck:

- All rotted or deteriorated wood shall be removed and replaced.

#### Poured Structural Concrete Deck:

The surface shall have a smooth and level finish and shall be free of excess moisture and loose debris.  Sharp ridges or other projections above the surface shall be removed before installation of roof assembly.

#### Precast/Prestressed Concrete Deck:

The surface shall have a smooth and level finish and shall be free of excess moisture and loose debris.  All joints between precast units shall be grouted with SikaGrout® 212 non-shrink, cementitious grout.  Any differentials in height between precast units shall be feathered for a smooth transition.  Sharp ridges or other projections above the surface shall be removed before installation of roof assembly.

###### WOOD NAILER INSTALLATION

### Install continuous wood nailers at the perimeter of the entire area and around projections and penetrations as shown on the Project Drawings.  Thickness shall be as required to match substrate and/or insulation height to allow a smooth transition.

### Nailers shall be anchored to resist a minimum force of 300 pounds per lineal foot (4,500 Newtons/lineal meter) in any direction.  Individual nailer lengths shall not be less than 3 feet (0.9 meter) long.  Nailer fastener spacing shall be at 12 inches (0.3 m) on center or 16 inches (0.4 m) on center if necessary to match the structural framing.  Fasteners shall be staggered 1/3 the nailer width and installed within 6 inches (0.15 m) of each end.  Two fasteners shall be installed at ends of nailer lengths.  Nailer attachment shall also meet the requirements of the current Factory Mutual Loss Prevention Data Sheet 1-49.

### Stainless steel, corrosion resistant, fasteners are required when mechanically attaching any Sika Sarnafil product to wood nailers and wood products treated with ACQ (Alkaline copper Quaternary). When ACQ treated wood is used on steel roof decks or with metal edge detailing, a separation layer must be placed between the metal and ACQ treated wood.

###### THERMAL BARRIER INSTALLATION (If Specified)

### Thermal barrier boards shall be laid over the substrate in parallel courses with end joints staggered and tightly butted.

### Thermal barrier boards shall be neatly cut to fit around penetrations and projections.

### When installed directly on fluted steel decks, the edge joints of the thermal barrier boards are to be parallel to and over deck ribs with end joints staggered.

### Do not install more thermal barrier board than can be covered with waterproofing membrane by the end of the day or the onset of inclement weather.

### Consult Product Data Sheet for additional installation instructions and information.

###### VAPOR BARRIER/AIR BARRIER INSTALLATION (If Specified)

[NOTE TO SPECIFIERS: IF REQUIRED, SELECT APPROPRIATE PRODUCT FROM THE CHOICES BELOW.]

˙

### Sarnavap-10

#### Sarnavap-10 is loose-laid over suitable substrate.

#### Overlap all edges 4 inches (100 mm) and seal with Sika Sarnafil Multi-Purpose Tape.

#### Extend Sarnavap-10 to perimeter and deck penetrations and seal to provide continuity of the     building's air/vapor envelope.

#### Sarnavap-10 must be sealed on the vertical surface at roof penetrations.

### Sarnavap Self-Adhered

#### Install Sarnavap Self-Adhered over a clean and dry substrate.  In concrete applications allow concrete to cure for at least 7 days.  Install in temperatures 32 degree F (0 degree C) and above.  The use of a primer is required on the following substrates: wood, concrete, lightweight concrete, gypsum boards and decks, and DensDeck® boards.  On metal decks use a metal plate (6 x 42 in. - 15 x 106 cm) to support the membrane end lap between metal flutes ensuring a complete end lap seal.

###### INSULATION INSTALLATION

### If tapered insulation is specified, install in accordance with approved tapered layout.

### Insulation shall be laid over the substrate in parallel courses with end joints staggered and tightly butted.

### Insulation shall be neatly cut to fit around penetrations and projections.

### Do not install more insulation board than can be covered with membrane by the end of the day or the onset of inclement weather.

### Use at least 2 layers of insulation when the total insulation thickness exceeds 2-1/2 inches (64 mm). Stagger joints at least 12 inches (0.3 m) between layers.

### Mechanical Attachment

#### Insulation shall be mechanically fastened to the deck with approved fasteners and plates at a rate according to the project requirements.  The insulation boards must rest evenly on the roof deck/substrate so that there are no significant air spaces between the boards and the substrate.  Each insulation board shall be installed tightly against the adjacent boards on all sides.

#### Fasteners are to be installed in accordance with fastener manufacturer's recommendations.  Fasteners are to have minimum penetration into structural deck recommended by the fastener manufacturer and Sika Sarnafil.

### Adhered Attachment

#### Adhere insulation boards to substrate using insulation adhesive.  Apply adhesive according to rates and procedures as directed in the application instructions for the product used.  All insulation boards must lay flat on the substrate.  Eliminate uneven surfaces to ensure positive contact between the insulation board and substrate.

###### VECTOR MAPPING GRID AND CONNECTIONS/COVER BOARD INSTALLATION

    - Lay the Vector Mapping Grid over the insulation. Follow Sika Sarnafil “Vector Mapping Grid (VMG) Installation Procedure.”

### Cover Board Installation

#### The cover board shall be adhered to the insulation, through the VMG, with insulation adhesive. Follow insulation adhesive application instructions.  Apply the adhesive to the insulation.  Place the cover board over the VMG and into the adhesive.  Ensure positive contact is made between the cover board and insulation board.

###### SURFACE CONDITIONER/PRIMER APPLICATION

[NOTE TO SPECIFIERS: SELECT THE APPROPRIATE PRODUCT FROM THE CHOICES BELOW, BASED ON ANTICIPATED APPLICATION TEMPERATURES AND APPLICABLE ENVIRONMENTAL REGULATIONS.]

˙

### Surface Conditioner 150

#### Apply Surface Conditioner 150 according to Sika Sarnafil’s printed instructions.

#### Surface Conditioner 150 must be applied to all horizontal and vertical concrete and masonry substrates to receive G476-15 SA membrane.  Do not apply surface conditioner to cover board substrates.

#### Do not apply surface conditioner during periods of inclement weather or when ambient or substrate temperatures are below 40 degree F (5 degree C). Temperature must be a minimum of 40 degree F (5 degree C) for surface conditioner application.

### Sarnavap Self-Adhered Primer

- Apply Sarnavap Self-Adhered Primer according to Sika Sarnafil’s printed instructions.

- Sarnavap Self-Adhered Primer must be applied to all horizontal and vertical concrete, masonry, and cover board substrates to receive G476-15 SA membrane when the ambient temperature or substrate temperature is below 40 degree F (5 degree C).

- Do not apply primer during periods of inclement weather or when ambient or substrate temperatures are below 25 degree F (-4 degree C).  Sarnavap Self-Adhered Primer is acceptable for use when application temperature is 25 degree F (-4 degree C) or higher.

###### SELF-ADHERED THERMOPLASTIC WATERPROOFING MEMBRANE INSTALLATION

- Comply with Sika Sarnafil’s most current installation instructions, specific recommendations and approved shop drawings for this project.

- Adhesion test strips are required prior to actual installation. Do not install G476 Self-Adhered membrane until a successful adhesion test has been completed. Refer to Sika Sarnafil’s most current quality assurance requirements for additional information.

- All surfaces shall be dry and free of dirt, dust, and debris.

- Apply Sarnafil self-adhered waterproofing membrane only in dry weather, and when the membrane, air, and substrate temperature is a minimum of 25 degree F (-4 degree C) over substrates primed with Sarnavap Self-Adhered Primer, a minimum of 40 degree F (5 degree C) over concrete or masonry substrates conditioned with Surface Conditioner 150, and a minimum 40 degree F (5 degree C) over unprimed cover boards.

- Workmen and all others that walk on the waterproofing shall wear clean, soft-soled shoes so as not to damage materials. Heed all manufacturer’s cautions and warnings in regard to product use.  Membrane is slippery when wet or covered with frost, snow and ice.  Take proper precautions.

- Lay out work to minimize traffic over installed areas.

###### HOT-AIR WELDING OF LAP AREAS

- General

#### All surfaces to be welded shall be clean and dry. No contaminants shall be present within lap areas.

#### Adjacent sheets shall be hot-air welded in accordance with Sika Sarnafil’s instructions.  All seams shall be hot air welded.  Lap area shall be a minimum of 2-1/2 inch (64 mm) wide.  Overlaps shall be with the flow of water where possible.

#### All cover strips and patches shall be G476, G410, or G459 membrane. Self-adhered membrane cannot be used as cover strips or patches.

#### A minimum 8 inch (20.3 cm) wide cover strip shall be used where membranes meet at end laps and all non-selvedge edges. Butt adjoining sheets closely, center the cover strip over both membranes and hot-air weld.

#### Patch all 3-way membrane overlaps (T joints) with a maximum 60 mil thick (1.5 mm), 4 inch (10.2 cm) round or square patch.

#### Welding equipment shall be provided by or approved by Sika Sarnafil.

- Machine Welding

#### Machine welded seams are achieved by the use of automatic welding equipment.  When using this equipment, Sika Sarnafil's instructions must be followed and local codes for electric supply, grounding and over current protection observed.  Dedicated circuit house power or a dedicated portable generator (30 A, 220 V, and recommended min. 7,500 Watts) is required.  No other equipment shall be operated off the generator.

- Quality Control of Welded Seams

All completed welded seams shall be checked by the waterproofing Applicator after cooling for continuity using a rounded screwdriver or other suitable blunt object.  On-site evaluation of welded seams shall be made daily by the Applicator.  Cross-section samples shall be taken a minimum of two times a day (AM/PM) through completed seams and evaluated immediately.  The samples must be dated and saved for evaluation by a Sika Sarnafil Technical Representative. Each test cut shall be patched by the Applicator.

###### MEMBRANE FLASHINGS

- All flashings shall be installed concurrently with the waterproofing membrane according to Sika Sarnafil approved details as the job progresses.  Flashings shall be adhered to compatible, dry, smooth, and solvent-resistant surfaces.  All masonry joints shall be struck flush. Rough or incompatible surfaces may be covered with minimum 1/2 inch (13 mm) CDX plywood. (Do not apply surface conditioner to plywood flashing substrates).  Flashing may be self-adhered waterproofing membrane or waterproofing membrane with field-applied adhesive installed according to Sika Sarnafil’s printed instructions.

- When adhering to vertical surfaces greater than 30 inches (76.2 cm) in height, provide intermediate fastening of the flashing membrane according to Sika Sarnafil requirements.

- Complete the entire waterproofing assembly and flashing in a single working day; avoid exposure of any components to rain, snow, or dew.  If rain threatens during the day, or in an emergency, protect the unfinished exposed waterproofing and flashing components.

- All flashing membranes shall be mechanically fastened along the top edge according to approved Sika Sarnafil details.  Acceptable fasteners shall be used to secure flashings to substrate. Seal top of termination with an acceptable sealant.

- All flashings shall extend a minimum of 8 inches (20.3 cm) above the overburden unless previously accepted by the owner’s representative and/or design professional and Sika Sarnafil, in writing.

- A minimum 8 inch (20.3 cm) wide cover strip shall be used where self-adhered flashing membranes meet at end laps, butt joints, and all non-selvedge edges. Butt adjoining sheets closely, center the cover strip over both membranes and hot-air weld. Complete inside and outside corner flashing details with prefabricated corner patches (Sarnacorners).

- No bituminous residue shall be in contact with the underside of the membrane flashing, unless self-adhered membrane or asphalt resistant membrane is used.  Flashing substrates contaminated with coal-tar shall be completely cleaned, or overlaid with minimum 1/2 inch (13 mm) thick CDX plywood or minimum 24 gauge stainless steel sheet metal.

###### SARNACLAD METAL FLASHINGS

- Complete all metal work in conjunction with waterproofing and flashings so that a watertight condition exists daily.

- Metal shall be installed to provide adequate resistance to bending and allow for normal thermal expansion and contraction.

- Metal joints shall be watertight.

- Metal flashings shall have a 4 inch (10.2 cm) minimum nailing flange and shall be fastened into solid wood blocking 4 inches (10.2 cm) on center staggered, or into concrete with acceptable concrete anchors 6 inches (15.2 cm) on center staggered.  Fasteners shall penetrate the wood nailer a minimum of 1-1/4 inch (31.7 mm) or into concrete a minimum of 1 inch (25.4 mm).

- Adjacent sheets of Sarnaclad metal shall be spaced 1/4 inch (6.3 mm) apart.  The end joints of the metal shall be fastened 6 inches (15.2 cm) on center.  The joints shall be covered with 2 inch (50.8 mm) wide aluminum tape.  A 4 inch (10.2 cm) wide membrane flashing strip shall be hot air welded over the joint.

###### TEMPORARY CUT-OFF

- All flashings shall be installed concurrently with the membrane in order to maintain a watertight condition as the work progresses. Provide temporary cut-offs around exposed edges and at incomplete flashing areas from the new membrane to the structural deck or existing waterproofing.  Remove the cut-offs completely before proceeding with subsequent work.

- If inclement weather occurs while a temporary cut-off is in place, the Applicator shall provide the labor necessary to monitor the situation to maintain a watertight condition.

###### water testing of drains and Electronic leak detection testing

- The Applicator shall water test the drains for a minimum of 16 hours prior to the EFVM test. Plug the drain and flood with enough water to completely cover the clamping ring the day before the EFVM test. The Sika Sarnafil Technical Service Representative attending the EFVM test will inspect for leakage upon his arrival to the jobsite.

- Perform EFVM electronic leak detection testing over the completed waterproofing membrane for testing of capillary defects and/or breaches in the membrane prior to the installation of subsequent layers.

- Should leaks be discovered, the Applicator shall locate leak source(s) and make repairs.  Re-test to assure watertightness.  All costs associated with the repairs shall be borne by the Applicator.

###### SEPARATION LAYER INSTALLATION (If Specified)

### Separation layer is installed above the membrane as additional protection as specified and according to the detail drawings lapping all edges a minimum of 4 inches (10.2 cm).

### Cut separation layer with scissors or utility blades.  Do not use hot air welding equipment to cut the separation layer.

###### DRAINAGE COMPOSITE INSTALLATION

[NOTE TO SPECIFIERS:  DELETE THIS SECTION IF DRAINAGE IS INCLUDED IN THE VEGETATIVE COVER SECTION.]

- Install drainage composite directly over the waterproofing membrane.  Install drainage composite immediately after Sika Sarnafil’s inspection and acceptance of the waterproofing installation.

Note:	When pre-vegetated modules are specified, Drainage Composite 3811R must be used. Install with grey polyester fabric facing the pre-vegetated module and the black polypropylene fabric facing the waterproofing membrane.

- Neatly trim drainage composite to fit closely around penetrations and at the base of all drains to ensure that water will flow freely from composite into drain openings.

- All cut edges of the drainage composite shall be covered in order to protect the waterproofing membrane from damage.

- Proceed with installation of vegetative cover promptly.

###### PRECAST CONCRETE PAVER SYSTEM (If Specified)

Prior to installation of the waterproofing system and pavers, inspect the deck for height and location of thresholds, stairs, and other major transition areas.

Precast Pavers on Pedestals:

#### Install pavers according to the manufacturer’s published instructions.

#### Cut and fit pavers neatly at the base of walls and projections.

#### Shim or adjust pavers as required to provide a level surface with no more than 1/8 inch (3.2 mm) height differential.

#### Pavers will be power washed or cleaned to remove any laitance, dirt, or other unacceptable surface irregularities.

###### MONITORING THE VEGETATIVE COVER INSTALLATION

- The waterproofing Applicator shall monitor vegetative cover installation to assure no damage is done to the waterproofing membrane.

- Alert all parties concerned of any activities that might damage or adversely affect the long-term performance of the waterproofing.

###### DETAILS

Refer to system specific details at http://usa.sarnafil.sika.com.